Contents

Rasterizing GADM Data for Uganda and Rwanda	2

Downloading GADM Data:	2

Converting Polygon to Line in QGIS:	3

Raster Creation:	4

Parameters for Raster Creation:	5

# Rasterizing GADM Data for Uganda and Rwanda

Steps to Rasterize the data from GADM Data is as follows:

## Downloading GADM Data:

- Open https://gadm.org/data.html for GADM Countrywide shapefiles.

- Click on Download Data by Country

- Select Country from the drop-down menu

- Download shapefiles in to a specific folder. Since we have level 4 for Uganda, we will be using level4 shape file for Rwanda.

- Extract the data from zipped folder.

## Converting Polygon to Line in QGIS:

- Open QGIS Application

- Add “gadm36_RWA_4.shp” file to layers section in QGIS Application. This is level four shape file.

- Since the shape file is of polygons

- We must convert this to line for creating raster.

- To convert polygon to lines in QGIS

  - Go to Vector tab

  - Select Geometry Tools

  - Select Polygon to Lines

- You will get a dialog box to execute the conversion. Do not change anything and run the command.

- After execution, we will get a temporary file(Layer) added to the layers section.

- We will be using this to create raster data.

## Raster Creation:

- To create a raster, go to “Raster” tab  Conversion  Rasterize (Vector to Raster)

- Update parameters for raster and run the program.

- You will get the raster file added to the Layers section and tiff file added to specified folder.

- Follow the same process for Uganda shape files as well.

## Parameters for Raster Creation:

- Resolution: 1/3600 : 0.0002777 : This is the fixed value to burn in Raster Parameters.

- Output Raster Size Units: Georeferenced Units

- Width/ horizontal Resolution: 0.00030 for 30 mts width

- Height/ vertical resolution: 0.00030 for 30 mts height

- Output Extent: Calculate from Layer and select the line file we have created

- Rasterized: Save data to a specific folder with required name.